ASSIGNMENT #02 – CLASS ACTIVITY

Actuators, Sensors and Signals – Fall Semester 2026

ROS 2 Based Ultrasonic Sensor and Servo Motor Control System

| Course | Actuators, Sensors and Signals |
| --- | --- |
| Submitted to | Prof. Muhammad Ilyas, Ph.D. in Robotics Engineering |
| Student | Kaldybekov Adilkhan |
| Platform | Arduino Uno + Ubuntu 24.04 + ROS 2 Jazzy |
| Final hardware | HC-SR04 ultrasonic sensor + SG90 servo motor |

School of IT & Engineering (SITE)

Kazakh-British Technical University

# 1. Assignment Requirements and Objective

The assignment requires the use of a studied actuator/device, an input sensor, and at least two ROS nodes. The ROS part must include a subscriber that receives sensor information and a publisher that sends a command to a device. The implemented project satisfies these requirements using an HC-SR04 ultrasonic sensor, an Arduino Uno, an SG90 servo motor, and two ROS 2 Python nodes.

## 1.1 Project Objective

The objective is to build a simple proximity-controlled actuator system. The ultrasonic sensor measures the distance to an object. The measured distance is transmitted from the Arduino to ROS 2 over USB serial communication. ROS 2 evaluates the distance and sends a servo angle command back to the Arduino. When an object is within 50 cm, the servo moves to 90°. When the object is farther than 50 cm, the servo returns to 0°.

## 1.2 Functional Requirements

Measure object distance using an ultrasonic sensor.

Transmit distance readings from Arduino Uno to a ROS 2 node.

Publish the sensor reading on the ROS topic /distance_cm.

Use a second ROS 2 node to subscribe to /distance_cm and apply the 50 cm decision threshold.

Publish the desired servo angle on /servo_angle.

Transmit the command back to Arduino Uno over USB serial.

Drive the SG90 servo to either 0° or 90° according to the ROS decision.

# 2. System Components

| Component | Type | Purpose | Interface |
| --- | --- | --- | --- |
| Arduino Uno | Microcontroller | Reads the ultrasonic sensor and controls the servo | USB serial, digital I/O, PWM |
| HC-SR04 | Ultrasonic sensor | Measures object distance | TRIG and ECHO digital signals |
| SG90 | Servo actuator | Provides angular motion | PWM control on D9 |
| Ubuntu 24.04.4 LTS | Operating system | Runs ROS 2 and the Python nodes | USB serial to Arduino |
| ROS 2 Jazzy | Middleware | Implements publisher/subscriber communication | /distance_cm and /servo_angle topics |

## 2.1 Why the SG90 Servo Was Used

The SG90 is a standard PWM-controlled micro servo and can be driven directly from an Arduino control pin using the Arduino Servo library. This makes it appropriate for demonstrating an actuator controlled by a ROS-generated command while keeping the hardware interface simple and reliable.

# 3. Hardware Circuit

Figure 1. Hardware wiring of HC-SR04, Arduino Uno and SG90 servo.

## 3.1 Connection Table

| Device pin | Arduino Uno pin | Function |
| --- | --- | --- |
| HC-SR04 VCC | 5V | Sensor power |
| HC-SR04 GND | GND | Sensor ground |
| HC-SR04 TRIG | D7 | Ultrasonic trigger pulse |
| HC-SR04 ECHO | D6 | Echo pulse input |
| SG90 signal | D9 | Servo PWM command |
| SG90 VCC | 5V | Servo power for laboratory test |
| SG90 GND | GND | Servo ground |

## 3.2 Ultrasonic Distance Formula

The HC-SR04 measures the round-trip travel time of an ultrasonic pulse. The distance is calculated using:

distance = duration × 0.0343 / 2

Here, duration is measured in microseconds. The value 0.0343 cm/µs is the approximate speed of sound in air. The result is divided by two because the sound travels from the sensor to the object and then back to the sensor.

# 4. Tinkercad Simulation Stage

Before the real prototype was connected to ROS 2, the sensor-actuator logic was verified in Tinkercad. The simulation used an ultrasonic distance sensor and a standard servo motor. The Arduino code measured distance and directly selected the servo position.

if (distance <= 50)
{
    servo.write(90);
}
else
{
    servo.write(0);
}

This first stage verified the electrical connections, the ultrasonic distance calculation, the 50 cm threshold, and the mechanical response of the servo. For the final ROS 2 implementation, this decision logic was removed from Arduino and moved into a ROS node.

# 5. Final Arduino Firmware

In the final design the Arduino acts as a hardware interface rather than the main decision-maker. It periodically measures the distance and sends a serial line in the format D:<distance>. It also listens for commands from ROS in the format S:<angle> and applies the received angle to the SG90.

| Direction | Serial message example | Meaning |
| --- | --- | --- |
| Arduino → ROS | D:61.50 | Measured distance is 61.50 cm |
| ROS → Arduino | S:90 | Move servo to 90° |
| ROS → Arduino | S:0 | Move servo to 0° |

A baud rate of 115200 bit/s was used for communication. The Arduino Serial Monitor must be closed during ROS operation because only one process should own the serial port at a time.

# 6. ROS 2 Architecture

Figure 2. ROS 2 publisher/subscriber architecture used in the project.

## 6.1 Node 1 – arduino_bridge

The arduino_bridge node is the hardware interface. It opens /dev/ttyUSB0 at 115200 baud, reads lines sent by Arduino, converts D:<distance> messages into ROS Float32 messages, and publishes them on /distance_cm. The same node subscribes to /servo_angle and writes S:<angle> back to Arduino.

## 6.2 Node 2 – distance_controller

The distance_controller node contains the decision logic. It subscribes to /distance_cm. If the received value is less than or equal to 50 cm, it publishes 90 on /servo_angle. Otherwise, it publishes 0. Repeated identical commands are suppressed using a last_angle variable.

## 6.3 ROS Topics

| Topic | Message type | Publisher | Subscriber |
| --- | --- | --- | --- |
| /distance_cm | std_msgs/Float32 | arduino_bridge | distance_controller |
| /servo_angle | std_msgs/Int16 | distance_controller | arduino_bridge |

# 7. ROS 2 Installation and Package Setup

The project was run on Ubuntu 24.04.4 LTS with ROS 2 Jazzy. The workspace and Python package were created with the following commands:

mkdir -p ~/ros2_ws/src
cd ~/ros2_ws/src
ros2 pkg create --build-type ament_python ultrasonic_servo --dependencies rclpy std_msgs

The package was built and sourced with:

cd ~/ros2_ws
colcon build --packages-select ultrasonic_servo
source ~/ros2_ws/install/setup.bash

The two nodes were started in separate terminals:

ros2 run ultrasonic_servo arduino_bridge
ros2 run ultrasonic_servo distance_controller

# 8. Experimental Procedure and Results

## 8.1 Serial Communication Test

The Arduino appeared in Ubuntu as /dev/ttyUSB0. A minicom test at 115200 baud confirmed that distance data were being transmitted correctly. Observed readings included values around 280 cm for a distant object and approximately 61 cm when an object was moved closer.

D:279.92
D:66.94
D:280.35
D:61.50
D:61.10

The reverse communication path was also tested by sending S:90 and S:0 from the computer to the Arduino. The SG90 responded correctly, confirming bidirectional serial communication.

## 8.2 ROS 2 Test

After starting arduino_bridge, the node continuously received distance values from the Arduino. Example ROS log output is shown below.

[INFO] [arduino_bridge]: Distance: 61.5 cm
[INFO] [arduino_bridge]: Distance: 61.1 cm
[INFO] [arduino_bridge]: Distance: 60.7 cm

## 8.3 Expected Final Behaviour

| Measured distance | ROS decision | Published angle | Servo state |
| --- | --- | --- | --- |
| > 50 cm | Object is outside threshold | 0° | Returned / closed position |
| ≤ 50 cm | Object detected inside threshold | 90° | Activated / open position |

The system therefore demonstrates the complete sensor → ROS decision → actuator loop required by the assignment. The main control condition is executed by ROS 2 rather than locally on the microcontroller.

# 9. Discussion

Separating hardware access from control logic makes the system modular. Arduino is responsible for timing-sensitive sensor and actuator interfacing, while ROS 2 handles communication and decision-making. The two ROS nodes can be replaced or extended independently without changing the physical wiring. For example, the threshold controller could later be replaced by a more advanced state machine, a GUI, or another sensor-processing node.

The use of ROS topics also makes the system observable during operation. Commands such as ros2 topic echo /distance_cm and ros2 topic echo /servo_angle can be used to inspect the information flow in real time.

# 10. Conclusion

A complete ROS 2 sensor-actuator system was implemented using an HC-SR04 ultrasonic sensor, Arduino Uno, and SG90 servo motor. The project was first validated as a direct Arduino/Tinkercad control system and then converted into a ROS 2 architecture. The Arduino transmits sensor data to ROS 2, the distance_controller node evaluates the 50 cm threshold, and the resulting servo command is sent back through the arduino_bridge node. The implementation demonstrates sensor acquisition, actuator control, serial communication, ROS publishers and subscribers, and a clear separation between hardware interfacing and high-level control.

# Appendix A – Arduino Uno Firmware

#include <Servo.h>

Servo servo;

const int trigPin = 7;
const int echoPin = 6;
const int servoPin = 9;

unsigned long lastMeasure = 0;

void setup()
{
  pinMode(trigPin, OUTPUT);
  pinMode(echoPin, INPUT);

  servo.attach(servoPin);
  servo.write(0);

  Serial.begin(115200);
}

void loop()
{
  if (millis() - lastMeasure >= 100)
  {
    lastMeasure = millis();

    digitalWrite(trigPin, LOW);
    delayMicroseconds(2);

    digitalWrite(trigPin, HIGH);
    delayMicroseconds(10);

    digitalWrite(trigPin, LOW);

    unsigned long duration = pulseIn(echoPin, HIGH, 30000);

    if (duration > 0)
    {
      float distance = duration * 0.0343 / 2.0;
      Serial.print("D:");
      Serial.println(distance);
    }
  }

  if (Serial.available())
  {
    String command = Serial.readStringUntil('\n');
    command.trim();

    if (command.startsWith("S:"))
    {
      int angle = command.substring(2).toInt();
      angle = constrain(angle, 0, 180);
      servo.write(angle);
    }
  }
}

# Appendix B – ROS 2 Node 1: arduino_bridge.py

import time
import serial
import rclpy
from rclpy.node import Node
from std_msgs.msg import Float32, Int16


class ArduinoBridge(Node):
    def __init__(self):
        super().__init__('arduino_bridge')

        self.serial_port = serial.Serial(
            '/dev/ttyUSB0', 115200, timeout=0.01
        )
        time.sleep(2)

        self.distance_pub = self.create_publisher(
            Float32, '/distance_cm', 10
        )

        self.servo_sub = self.create_subscription(
            Int16, '/servo_angle', self.servo_callback, 10
        )

        self.timer = self.create_timer(0.02, self.read_arduino)
        self.get_logger().info('Arduino bridge started')

    def read_arduino(self):
        if self.serial_port.in_waiting > 0:
            line = self.serial_port.readline()

            try:
                line = line.decode().strip()
            except UnicodeDecodeError:
                return

            if line.startswith('D:'):
                try:
                    distance = float(line[2:])
                    msg = Float32()
                    msg.data = distance
                    self.distance_pub.publish(msg)
                    self.get_logger().info(
                        f'Distance: {distance:.1f} cm'
                    )
                except ValueError:
                    pass

    def servo_callback(self, msg):
        angle = max(0, min(180, msg.data))
        command = f'S:{angle}\n'
        self.serial_port.write(command.encode())
        self.get_logger().info(
            f'Servo command: {angle} degrees'
        )


def main(args=None):
    rclpy.init(args=args)
    node = ArduinoBridge()

    try:
        rclpy.spin(node)
    except KeyboardInterrupt:
        pass
    finally:
        node.serial_port.close()
        node.destroy_node()
        rclpy.shutdown()


if __name__ == '__main__':
    main()

# Appendix C – ROS 2 Node 2: distance_controller.py

import rclpy
from rclpy.node import Node
from std_msgs.msg import Float32, Int16


class DistanceController(Node):
    def __init__(self):
        super().__init__('distance_controller')

        self.threshold = 50.0
        self.last_angle = None

        self.distance_sub = self.create_subscription(
            Float32,
            '/distance_cm',
            self.distance_callback,
            10
        )

        self.servo_pub = self.create_publisher(
            Int16,
            '/servo_angle',
            10
        )

        self.get_logger().info('Distance controller started')

    def distance_callback(self, msg):
        distance = msg.data

        if distance <= self.threshold:
            angle = 90
        else:
            angle = 0

        if angle != self.last_angle:
            command = Int16()
            command.data = angle
            self.servo_pub.publish(command)

            self.get_logger().info(
                f'Distance: {distance:.1f} cm -> Servo: {angle} deg'
            )

            self.last_angle = angle


def main(args=None):
    rclpy.init(args=args)
    node = DistanceController()

    try:
        rclpy.spin(node)
    except KeyboardInterrupt:
        pass
    finally:
        node.destroy_node()
        rclpy.shutdown()


if __name__ == '__main__':
    main()

# References

1. Assignment #02 – Class Activity, Actuators, Sensors and Signals, Fall Semester 2026, Prof. Muhammad Ilyas.

2. ROS 2 Jazzy documentation – ROS 2 concepts, nodes, topics, publishers and subscribers.

3. Arduino Servo library documentation – standard hobby servo control on Arduino Uno.

4. HC-SR04 ultrasonic sensor technical documentation – ultrasonic ranging principle and trigger/echo timing.